LDATA

PURPOSE :       LDATA reads an FCS histogram file and writes its raw data

segment to the screen or to an ASCII file.  It is used when

it is convenient to have histogram data in a human readable

format, or when the data needs to be read by a standard

analysis or statistics program.

USAGE :         LDATA <inputfile> [<outputfile>]

SYNOPSIS :      The output format from a one parameter histogram is simply

each channel count in text form, separated by a single space,

with 16 values per line.  A 256 channel histogram therefore

produces 16 lines of output, while a 1024 histogram produces

64 lines.

A two parameter histogram is written with one row of channels

per row of output.  This puts the origin of the histogram at

the top left of the screen or output file and the first

parameter on the x-axis and the second on the x-axis.  Values

are separated by single space characters.  This means that

for standard 64 channel histograms with fewer than 65536

events per channel, each line might be as long as

(5 + 1) * 64 = 384 characters.

If no output file is specified, output will be written to

the computer screen.